The primary governing standard for commercial and industrial valves used in ASME piping systems is ASME B16.34.

This standard establishes the pressure-temperature ratings, dimensions, materials, and testing requirements for cast, forged, and fabricated valves with flanged, threaded, and welding ends.

Core ASME Valve Standards

Depending on the specific valve type and application, several specialized standards apply:

- ASME B16.34 (Valves — Flanged, Threaded, and Welding End): The overall parent standard for pressure ratings and design criteria.

- API 600 / API 602: Governs steel gate valves (bolted bonnets and compact forged designs) heavily used in petroleum and process piping.

- API 598: The absolute benchmark standard for valve inspection and pressure testing.

- API 6D: Establishes the design requirements for pipeline valves used in transportation systems (matching ASME B31.4 and B31.8).

- ASME B16.10: Standardizes face-to-face and end-to-end dimensions ensuring different valve brands fit into the exact same physical space in a pipeline.

Quick Reference by Valve Type

| Valve Type | Primary Design Standard | Face-to-Face Dimension Standard |
| --- | --- | --- |
| Gate Valves | API 600 / API 602 / ASME B16.34 | ASME B16.10 |
| Globe Valves | BS 1873 / API 623 / ASME B16.34 | ASME B16.10 |
| Check Valves | API 594 / BS 1868 | ASME B16.10 |
| Ball Valves | API 608 / ISO 17292 | ASME B16.10 |
| Butterfly Valves | API 609 | ASME B16.10 / API 609 |

For purchasing and reviewing official compliance guidelines, check the official ASME B16.34 Product Page.

If you want to pin down the exact compliance requirements, tell me:

- What type of valve are you using (e.g., ball, gate, globe, butterfly)?

- What is the piping system class or operating pressure/temperature?

I can provide the exact section, rating table, or wall thickness formula you need.